|  | Attachment 10 SOURCES OF PM2.5 COMPONENTS OF PM2.5,THEIR SOURCE AND FUTURE PROJECTIONS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | The figure below shows a west to east traverse of 111 km. across London from Henly to |  |  |  |  |  |  |  |
|  | Southend-on-Sea giving the Annual Mean PM2.5 concentration and the breakdown of |  |  |  |  |  |  |  |
|  | components. |  |  |  |  |  |  |  |
|  | It shows a remarkably uniform "background" concentration between 10-12 ug/m^3 with |  |  |  |  |  |  |  |
|  | little direct urban traffic contribution at either extremity. Towards central London, the mean |  |  |  |  |  |  |  |
|  | concentration increases to between 14-16 ug/m^3, the increase mainly traffic related, |  |  |  |  |  |  |  |
|  | suggesting a 30-40 % contribution to the overall PM2.5 mean. |  |  |  |  |  |  |  |
|  | The PM2.5 values for Village road, though scattered, suggest a "background" concentration |  |  |  |  |  |  |  |
|  | of 8.3 ug/m^3 (corresponding to zero traffic flow) rising to 11.5 ug/m^3 for a traffic flow of |  |  |  |  |  |  |  |
|  | 600 vehicles per hour, suggesting a similar traffic-related contribution of 30-40%. |  |  |  |  |  |  |  |
|  |  |  |  |  |  | traffic sources |  |  |
|  |  |  |  |  |  | non-traffic sources |  |  |
|  |  |  |  |  |  | urban dust |  |  |
|  |  |  |  |  |  | rural dust |  |  |
|  |  |  |  |  |  | secondary organic |  |  |
|  |  |  |  |  |  | secondary inorganic |  |  |
|  |  |  |  |  |  | residual |  |  |
|  |  |  |  |  |  | sea salt |  |  |
|  | CONTRIBUTIONS AND PROJECTIONS |  |  |  |  |  |  |  |
|  | The Table below taken from (1) shows the contributions to annual mean PM2.5 in the UK in |  |  |  |  |  |  |  |
|  | 2009 and a projection to 2020 indicating a 15% reduction. |  |  |  |  |  |  |  |
|  |  | Component | 2009 | 2020 |  |  |  |  |
|  |  | sea salt | 0.67 | 0.67 |  |  |  |  |
|  |  | residual | 1 | 1 |  |  |  |  |
|  |  | secondary inorganic aerosol | 4.05 | 3.34 |  |  |  |  |
|  |  | secondary organic aerosol | 0.86 | 0.86 |  |  |  |  |
|  |  | regional primary | 1.14 | 0.8 |  |  |  |  |
|  |  | rural dust | 0.51 | 0.51 |  |  |  |  |
|  |  | urban dust | 0.62 | 0.62 |  |  |  |  |
|  |  | point source | 0.07 | 0.06 |  |  |  |  |
|  |  | non-traffic area sources | 1.02 | 0.78 |  |  |  |  |
|  |  | traffic area sources | 0.75 | 0.38 |  |  |  |  |
|  |  | Total | 10.69 | 9.02 |  |  |  |  |
|  | (1) Fine Particulate Matter(PM2.5) in the United Kingdom, Air Quality Expert Group, Prepared for |  |  |  |  |  |  |  |
|  | Department for Environment,Food and Rural Affairs, Scottish Executive;Welsh Government, and |  |  |  |  |  |  |  |
|  | Department of the Environment in Northern Ireland (2012); http://www.defra.gov.uk/environment/ |  |  |  |  |  |  |  |
|  | quality/air/ |  |  |  |  |  |  |  |