|  | Attachment 8 SUMMARY OF VILLAGE ROAD AIR QUALITY MEASUREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Background |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sixteen(16) periods of 1 hour duration between 8.00 am and 4.30pm within the interval |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Aug.1st to Nov.24th were selected to enable a wide range of hourly traffic flows from 50 - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 600 vehicles to be studied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The concentrations of major pollutants such as nitrogen dioxide(NO2), micro-particulates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | of >= 10 microns(PM10) and <= 2.5 microns(PM2.5) were measured by the portable Plume |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lab Flow device. The accuracy of this miniature device was checked for NO2,PM10 & PM2.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | concentrations against the Boughton Continuous NO2 Monitor and the Wrexham AURN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Monitor, which reports Hourly Averages of all three pollutants, on several occasions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Plots were prepared of the concentrations of NO2,PM10 & PM2.5 versus the total hourly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | traffic flow through the centre of the village (ie., North+South) and compared with World |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Health(WHO) and EU(UK) allowable exposure limits (see Attachments 1-3 for a comparison |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | of the variation of standards between the WHO, EU and EPA (USA). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All three pollutant concentrations increased significantly with traffic flow, despite the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | expected scatter, with NO2 and PM10 showing the largest increases. The best straight line fit of concentrations for traffic volumes from 0 to 600v/hr are plotted on the graphs below and are compared with the full range of values(attachment 5) in the following table ug/m3 Full range Straight line fit values NO2 0 – 74 5 – 25 PM10 8 - 85 9 - 32 PM2.5 2 - 64 8.5 -11.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Conclusions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NO2: | the concentrations lie well below the recommended annual mean exposures of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40ug/m^3{EU(UK) & WHO} to 100ug/m^3(USA). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NO2 does not appear to be a significant concern |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PM10: | the concentrations of PM10 tend to lie below the WHO & EU(UK)24 hour mean |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | exposure limit of 50 ug/m^3. This limit, though is being approached transiently |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | at peak traffic flows during school term. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Particular attention should be paid to PM10 concentrations during peak |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | traffic flow over the Winter Period, when domestic production of PM10's is |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | expected to increase |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PM2.5: | the concentrations of PM2.5 on average are already close to the WHO and EPA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (USA) annual mean exposure limits of 10 ug/m^3 and 12 ug/m^3,respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | On several occasions they have exceeded these limits by a factor 1.5-2! However, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | they still lie well below the EU(UK) annual mean standard of 25ug/m^3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | As with PM10 they are expected to increase over the Winter period. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | PM2.5 concentrations are a cause for concern! |  |  |  |  |  |  |  |  |  |  |  |  |  |  |